Jimmy Tran

Bourgeois

CSC 4222

24 October 2017

Assignment 4

1. Jill’s network can be ARP spoofed or poisoned, hit with a ping of death, smurfed, IP spoofed, and packet sniffed among many other threats. With an open connection like this, her network is pretty much vulnerable to any type of malicious attack really since there is virtually no security present to stop attackers.

2. The Ethernet frame devotes at least 14 bytes to the header with an additional 4 being optional. 6 each are required for the MAC source IP address and for the MAC destination IP address. An additional 4 bytes can be used for the 802.1Q tag and then 2 are required for the Ethertype or length section.

3. Assuming you know what the private secret key Ks is, you could send a message to the server asking for a reply that uses K for encryption. As the attacker would not know the server’s private key, the attacker could not reply while the server would, confirming that you share K with the server and the key is not only being shared with a man-in-the-middle.

If you don’t know Ks, then you can send a message using the server’s public key for encryption. The message would contain K and ask for a reply with the server’s private key. The man-in-the-middle might be able to intercept and decrypt the message but would not be able to reply with the private key. The server would probably send its private key back, confirming that you now share K with the server even if you don’t know if a man-in-the-middle exists.

4. A DOS on a web server can be performed by sending many packets faster than the web server can process in a small window of time, which should be easy with more than 64,000 IP addresses available for use. This can be prevented by using SYN cookies which have the server add connections to a connection table after verifying cookie information instead of just allocating space on the connection table immediately.

5. With 65536 possible transaction IDs: (65536+1024)*.01=665.6. This means he must trigger 666 (rounded up from 665.6) requests for one of his to compromise the DNS cache.

6. The birthday attack equation can be used for this problem. To get a 50% chance at succeeding, the attacker needs to send 2 * 232 * ln(2) = 77160 DNS requests and responses each.

7. Modern OSes have taken measures such as sending a RST response regardless of the port state in response to FIN packets. An IDS could detect a FIN scan by checking what a FIN packet’s source is or at least by checking that the port that the FIN packet was sent to was actually open for wherever the FIN packet came from rather than just sending a response immediately. A packet lacking a source or that claims it was communicating with a closed or inactive port is probably an issue that needs to be checked.

8. For the ACK response, the sequence number is 156955004 and the acknowledgement number is 883790340.

9. If the third party can sniff packets, he likely already knows the SYN and ACK sequence for both other parties. As such, all the third party must do is send a packet containing the reset bit to one or both parties with the SYNs and ACKs following their proper pattern to make them believe the other sent the packet.

If the third party cannot sniff packets, he would have to brute force his way through by sending many packets with the reset bit and guessing at SYN and ACK numbers until at least one of the two others accepted one of his packets. Only one of them needs to drop the connection as the connection will no longer be acknowledged at that point and all further packets received would be ignored.

10. A good modern firewall has an anti-spoofing rule that ensures outgoing packets have a source IP address that is the same as the Security Gateway interface. Packets that come to the external interface with spoofed internal IP addresses are blocked as a result as they do not match the Security Gateway interface even if they have a “correct” internal IP address. In short, the external interfaces of a network do not accept packets with internal source IP addresses.